Wrong Position

The tower is placed in wrong position

Other examples

The placement of the printed model should be in center of the bed so :

- The center of the model in the center of the bed

- In code we have corner and center , when running code we use center, how

In G-code, coordinates can be referenced from either the corner or the center—it depends on how the machine’s origin (0,0) is set. G-code itself does not force one method.

Most CNC machines and many 3D printers use the front-left corner of the bed/workpiece as X0 Y0.

Example

G0 X0 Y0

➡ The tool moves to the front-left corner.

G1 X100 Y50

Move 100 mm right and 50 mm back from that corner.

Typical setup:

- X0 Y0 → front-left corner

- X increases → right

- Y increases → toward the back

So it has to adapt that we can know whether the G-code uses corner origin or center origin, but you usually determine it by checking the machine setup and the G-code values

Check the Machine Home Position via G28
Look at the Coordinates in the G-code

All values positive → usually corner origin.

Check the CAM / Slicer Settings

Check Work Coordinate Commands

G92 can redefine the current position as (0,0).

Maybe we need to update code to have Printer profile

Filament issues:

Extra unwanted filament , maybe because filament render prints the motion of nozzle not what g-code tells

Deformed filaments because of several factors such as :

- Speed of printing, layerHeight , maybe delay but I don’t think so