Good idea. If you want a Three.js digital twin of a Creality-style FDM printer, the best reference is something like the Creality Ender-3, because it has a simple and well-known architecture. Its core mechanical parts include the print bed, X/Y/Z axes, extruder/hotend, frame, motors, and spool holder, which makes it ideal for simulation.

I’ll show you how to:

- Define the realistic printer hierarchy

- Map preventive-maintenance sensors

- Create a Three.js scene in Vite

- Simulate axis movement + telemetry hooks

1️⃣ Realistic FDM Printer Structure (Creality-like)

Use this hierarchy in Three.js (important for animation).

Printer
│
├── Frame (Skeleton)
│   ├── Base
│   ├── Vertical Columns
│   └── Top Gantry
│
├── Motion System
│   ├── X Axis
│   │   ├── X Rail
│   │   ├── X Carriage
│   │   └── Hotend Assembly
│   │
│   ├── Y Axis
│   │   ├── Y Rail
│   │   └── Bed Carriage
│   │
│   └── Z Axis
│       ├── Z Lead Screw
│       ├── Z Motors
│       └── Gantry Lift
│
├── Extrusion System
│   ├── Extruder Motor
│   ├── Bowden Tube
│   └── Hotend
│        ├── Heatsink
│        ├── Heatbreak
│        ├── Heater Block
│        └── Nozzle
│
├── Cooling
│   ├── Hotend Fan
│   └── Part Cooling Fan
│
├── Print Bed
│   ├── Heated Bed
│   ├── Bed Springs
│   └── Build Plate
│
├── Filament System
│   ├── Filament Spool
│   ├── Spool Holder
│   ├── Filament Sensor
│   └── Bowden Tube
│
└── Electronics
    ├── Power Supply
    ├── Mainboard
    ├── Screen
    └── Endstop Switches

Those elements correspond to the real printer components like stepper motors, hotend, belts, spool holder, and limit switches used in Creality printers.

2️⃣ Add Preventive-Maintenance Sensors

Since your project is DT + PMx + OctoPrint, attach sensors to the model.

Example mapping:

| Part | Sensor | Purpose |
| --- | --- | --- |
| Hotend | Thermistor | temperature anomaly |
| Heatsink | DS18B20 | cooling failure |
| Bed | Thermistor | heating stability |
| X carriage | ADXL345 | vibration detection |
| Extruder | Filament encoder | slip detection |
| Filament path | runout sensor | filament break |
| Frame | IMU | structural vibration |
| Electronics | current sensor | PSU issues |
| Nozzle | camera | spaghetti detection |

3️⃣ Vite + Three.js Project Setup

Since you use Vite 4.4.5 vanilla:

npm create vite@4 printer-dt
cd printer-dt
npm install three

Project:

src/
 ├ main.js
 ├ printer/
 │   ├ printer.js
 │   ├ bed.js
 │   ├ hotend.js
 │   ├ frame.js
 │   └ axes.js
 └ sensors/
     └ telemetry.js

4️⃣ Base Three.js Scene

main.js

import * as THREE from "three"
import { createPrinter } from "./printer/printer"

const scene = new THREE.Scene()

const camera = new THREE.PerspectiveCamera(
  60,
  window.innerWidth / window.innerHeight,
  0.1,
  1000
)

camera.position.set(4,4,6)

const renderer = new THREE.WebGLRenderer({antialias:true})
renderer.setSize(window.innerWidth, window.innerHeight)
document.body.appendChild(renderer.domElement)

const light = new THREE.DirectionalLight(0xffffff,1)
light.position.set(5,10,5)
scene.add(light)

scene.add(new THREE.AmbientLight(0x404040))

const printer = createPrinter()
scene.add(printer)

function animate(){
  requestAnimationFrame(animate)
  renderer.render(scene,camera)
}

animate()

5️⃣ Printer Model (Hierarchy)

printer/printer.js

import * as THREE from "three"
import { createBed } from "./bed"
import { createHotend } from "./hotend"

export function createPrinter(){

    const printer = new THREE.Group()

    const frame = new THREE.Mesh(
        new THREE.BoxGeometry(4,4,4),
        new THREE.MeshStandardMaterial({wireframe:true})
    )

    printer.add(frame)

    const bed = createBed()
    printer.add(bed)

    const hotend = createHotend()
    hotend.position.set(0,1,0)

    printer.add(hotend)

    printer.userData = {bed, hotend}

    return printer
}

6️⃣ Example Bed (Y-axis motion)

export function createBed(){

    const geometry = new THREE.BoxGeometry(2.5,0.1,2.5)

    const material = new THREE.MeshStandardMaterial({
        color:0x333333
    })

    const bed = new THREE.Mesh(geometry,material)

    bed.position.y = 0.2

    return bed
}

7️⃣ Axis Motion Simulation

Example:

printer.userData.hotend.position.x = Math.sin(time)*1.2
printer.userData.bed.position.z = Math.cos(time)*1.2

You now simulate X/Y printing movement.

8️⃣ OctoPrint Telemetry Integration

Later you connect:

OctoPrint → WebSocket → Three.js

Example telemetry packet:

{
 nozzleTemp: 210,
 bedTemp: 60,
 x: 120,
 y: 80,
 z: 0.3,
 vibration: 0.02
}

Then update the model:

hotend.position.x = data.x
bed.position.z = data.y
gantry.position.y = data.z

9️⃣ For Realistic Models

Download real models:

Sources:

- Printables

- Thingiverse

- GrabCAD

Search:

Ender 3 STEP
Ender 3 STL
Creality Ender CAD

Then load using:

GLTFLoader

🔟 Digital Twin Extensions (VERY IMPORTANT)

Add:

AI anomaly indicators

Example:

if(vibration > threshold)
   hotend.material.color.set(0xff0000)

Heatmap

color = mapTemperatureToColor(temp)

Camera overlay

texture = webcamStream

🔥 My recommendation for your project

For a PFE Digital Twin + Preventive Maintenance:

Simulate:

1️⃣ Frame
2️⃣ Bed (Y axis)
3️⃣ X carriage + hotend
4️⃣ Z gantry
5️⃣ Filament spool
6️⃣ Fans
7️⃣ Sensors

This already gives a complete digital twin.

✅ If you want, I can also generate:

- A full Creality Ender-3 Three.js digital twin architecture

- ready-to-run Vite project

- sensor visualization

- AI anomaly indicators

- OctoPrint WebSocket integration